## Lab 10: Microarchitectural Side-Channel Analysis

### Objectives

- Understand the timing differences between CPU Cache hits and Cache misses.

- Observe the performance penalty of Branch Misprediction in modern x86 processors.

- Learn how to use the Time Stamp Counter (TSC) for high-precision microarchitectural auditing.

### Part 1: Cache Timing Sensor

In this section, you will use cache_timing_sensor.c to measure the "gap" between a piece of data residing in the L1 cache versus data that must be fetched from main memory (RAM).

#### Key Functions to Investigate

- _mm_clflush(address): This intrinsic forces the CPU to evict a specific line from all levels of the cache.

- read_tsc_begin() and read_tsc_end(): These functions use the rdtsc instruction and lfence to ensure the CPU doesn't reorder instructions, which would ruin the timing measurement.

#### Instructions

- Compile and Run: Compile cache_timing_sensor.c using your platform's compiler (GCC/Clang or MSVC).

- Calibration: The program will attempt to calibrate your CPU's frequency using calibrate_cycles_per_second.

- Adjusting for Speed: If your machine has a high noise floor, you may need to increase the EXPERIMENTS constant to get a cleaner median.

#### Deliverables & Analysis

- Report: Provide the average/median hit time and miss time from your machine. Support it with a screenshot showing local computer time/date.

- Analysis: Why is the Miss/Hit slowdown usually significant (often 10x or more)?

- Analysis: Why do we use _mm_lfence() before and after reading the time stamp?

### Part 2: Branch Predictor Audit

In this section, you will use branch_predictor_audit.c to measure the cost of "surprising" the CPU's branch prediction hardware.

#### Key Functions to Investigate

- run_training_window(): This function runs a loop 100 times, passing true for the first 99 iterations to "train" the predictor.

- measure_window(bool mismatch_on_last_branch): This measures how long the 100-iteration window takes when the 100th branch matches the pattern versus when it breaks it.

#### Instructions

- Compile and Run: Compile branch_predictor_audit.c.

- Training Pattern: Observe the code; it repeats the "99 hits, 1 possible miss" pattern across many repeated_windows.

- The Signal: The "Mismatch Penalty" represents cycles wasted flushing the pipeline due to incorrect speculation.

#### Deliverables & Analysis

- Report: Provide your observed Median mismatch penalty. Support it with a screenshot showing the local computer time/date.

- Analysis: If the branch predictor was 100% accurate, what would the mismatch penalty be?

- Analysis: How could an attacker use this timing difference to determine the value of a secret boolean variable in a victim program?

### Troubleshooting Note

- Frequency Scaling: Ensure your laptop is plugged in and set to "High Performance" to avoid CPU throttling.

- Virtualization: If results show 0.00 cycles or massive noise, you may be in a VM where the rdtsc instruction is being intercepted by a hypervisor.

### Grading

- Part 1 50/50

  - Report with valid screenshot 20/50

  - Analysis 30/50

- Part 2 50/50

  - Report with valid screenshot 20/50

  - Analysis 30/50